#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <Wire.h>

Adafruit_MPU6050 mpu;

const int BUTTON_PIN = 18;

const int BUZZER_PIN = 19;

const float ACCEL_FALL_THRESHOLD = 20.0;

const float GYRO_FALL_THRESHOLD = 3.5;

bool fallDetected = false;

unsigned long fallTime = 0;

unsigned long lastBuzzTime = 0;

unsigned long lastPrintTime = 0;

bool buzzerState = false;

const unsigned long CONFIRM_WINDOW = 2000;

const unsigned long BUTTON_IGNORE_WINDOW = 10;

void setup() {

Serial.begin(115200);

while (!Serial) delay(10);

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(BUZZER_PIN, OUTPUT);

noTone(BUZZER_PIN);

Serial.println("Fall Detection System Started");

if (!mpu.begin()) {

Serial.println("Failed to find MPU6050 chip");

while (1) delay(10);

}

Serial.println("MPU6050 Found!");

mpu.setAccelerometerRange(MPU6050_RANGE_16_G);

mpu.setGyroRange(MPU6050_RANGE_2000_DEG);

mpu.setFilterBandwidth(MPU6050_BAND_21_HZ);

delay(100);

}

void loop() {

unsigned long currentTime = millis();

sensors_event_t a, g, temp;

mpu.getEvent(&a, &g, &temp);

float accelMag = sqrt(

a.acceleration.x * a.acceleration.x +

a.acceleration.y * a.acceleration.y +

a.acceleration.z * a.acceleration.z

);

float gyroMag = sqrt(

g.gyro.x * g.gyro.x +

g.gyro.y * g.gyro.y +

g.gyro.z * g.gyro.z

);

// Print sensor values only once per second instead of every loop

if (currentTime - lastPrintTime >= 1000) {

Serial.print("AccelMag: "); Serial.print(accelMag);

Serial.print(" | GyroMag: "); Serial.println(gyroMag);

lastPrintTime = currentTime;

}

// Detect fall

if (!fallDetected) {

if (accelMag > ACCEL_FALL_THRESHOLD || gyroMag > GYRO_FALL_THRESHOLD) {

fallDetected = true;

fallTime = currentTime;

lastBuzzTime = currentTime;

buzzerState = false;

Serial.println("==================================");

Serial.println(">> POSSIBLE FALL DETECTED!");

Serial.println(">> Press button to cancel if false alarm.");

Serial.println("==================================");

}

}

if (fallDetected) {

// Check button - reduced ignore window to 200ms

if (currentTime - fallTime > BUTTON_IGNORE_WINDOW) {

if (digitalRead(BUTTON_PIN) == LOW) {

Serial.println("==================================");

Serial.println(">> FALSE ALARM - Alert cancelled.");

Serial.println("==================================");

fallDetected = false;

noTone(BUZZER_PIN);

buzzerState = false;

return;

}

}

// Countdown

static unsigned long lastCountTime = 0;

if (currentTime - lastCountTime >= 1000) {

unsigned long elapsed = currentTime - fallTime;

if (elapsed < CONFIRM_WINDOW) {

Serial.print(">> Time remaining: ");

Serial.print((CONFIRM_WINDOW - elapsed) / 1000);

Serial.println(" seconds");

}

lastCountTime = currentTime;

}

// 10 second window expired

if (currentTime - fallTime >= CONFIRM_WINDOW) {

Serial.println("==================================");

Serial.println("        FALL DETECTED!           ");

Serial.println("==================================");

fallDetected = false;

noTone(BUZZER_PIN);

buzzerState = false;

return;

}

// Buzzer beep every 500ms

if (currentTime - lastBuzzTime >= 500) {

buzzerState = !buzzerState;

if (buzzerState) {

tone(BUZZER_PIN, 1000);

} else {

noTone(BUZZER_PIN);

}

lastBuzzTime = currentTime;

}

}

}